Wheelchair and Aircraft Compatibility Standards – Research Findings

1. Purpose

This research identifies the key information required to assess whether a passenger’s wheelchair or mobility aid can be safely transported on an aircraft. The findings will help the development team decide which wheelchair, battery and aircraft compatibility fields should be stored in the YAF database.

2. Wheelchair Dimensions

According to IATA guidance and the 2026 Mobility Aids Battery Checklist for battery-powered wheelchairs and mobility aids, airlines should collect the wheelchair or mobility aid’s manufacturer, model, total weight, and overall length, width and height, including accessories, as presented for air travel. These dimensions are important because they allow the airline to determine whether the mobility aid can pass through the aircraft door and fit inside the cargo compartment. IATA also recommends identifying removable or adjustable parts, such as the headrest, joystick or seat cushion, because removing or adjusting these components may reduce the wheelchair’s transport dimensions and improve aircraft compatibility.

There is no single standard wheelchair size, as dimensions vary depending on the manufacturer and model. Therefore, the YAF database should store model-specific wheelchair dimensions rather than using a fixed standard size.

Recommended Database Fields

- Manufacturer

- Model

- Wheelchair/mobility aid type

- Length (cm)

- Width (cm)

- Height (cm)

- Total weight (kg)

- Transport/folded dimensions

- Removable parts

- Adjustable parts

- Notes on transport configuration

Why This Information Is Needed

The wheelchair dimensions can later be compared with aircraft cargo-door and cargo-compartment dimensions to determine whether a particular wheelchair can be accommodated on a specific aircraft. If the wheelchair is too large, the airline may need to consider a different aircraft or alternative routing.

3. Battery Requirements

Battery type is an important factor when determining how a powered wheelchair or mobility aid can be transported by air. According to the IATA 2026 guidance, different requirements apply depending on the type of battery installed.

The main battery categories include:

- Non-spillable wet batteries — must comply with IATA Special Provision A67.

- Nickel-metal hydride batteries — must comply with Special Provision A199.

- Dry batteries — must comply with Special Provision A123.

- Spillable wet batteries — require additional handling because they contain corrosive liquid and should normally remain upright during loading, transport and unloading.

- Lithium-ion batteries — must comply with UN Manual of Tests and Criteria, Part III, subsection 38.3 (UN 38.3).

For lithium-powered mobility aids, batteries may remain installed when they are securely attached, protected against damage and the electrical circuits can be isolated. If lithium batteries are removed, they must be protected against short circuit and transported according to IATA requirements. Mobility aids containing more than 300 Wh of installed lithium batteries may require additional safety consideration by the airline.

The database should also record how the wheelchair can be made electrically safe for transport, including instructions for isolating the battery and preventing accidental activation.

Recommended Database Fields

- Battery type

- Battery capacity / total Watt-hours (Wh), where applicable

- Number of installed batteries

- UN 38.3 compliant — Yes/No

- Applicable battery standard — A67 / A199 / A123

- Battery removable — Yes/No

- Battery removal required for transport — Yes/No

- Number of spare batteries

- Spare battery Watt-hour rating

- Power isolation / circuit disconnection instructions

- Short-circuit protection information

- Freewheel mode instructions

- Battery handling requirements

- Battery transport notes

Why This Information Is Needed

Recording this information will help the YAF system and airline staff identify the correct handling requirements for each battery-powered mobility aid before travel. It can also help reduce safety risks and avoid delays caused by missing battery information.

4. Aircraft Door Compatibility

Aircraft compatibility depends on whether a wheelchair or mobility aid, in its air-travel configuration, can physically pass through the aircraft cargo door and be safely accommodated for transport.

IATA recommends that the mobility aid be adjusted to its lowest practical height and that its length, width and height be checked against the aircraft scheduled to operate the passenger’s journey. Booking staff should have access to aircraft dimension information so this compatibility can be checked before travel.

IATA also states that airlines should provide information about cargo compartment door dimensions and aircraft weight limitations, because these may prevent some mobility aids from being transported on particular aircraft types.

During preparation for air travel, the mobility aid’s maximum length, width and height should be verified, and its height should be reduced as much as possible so that it can fit through the cargo door.

Therefore, the YAF database should store aircraft-specific cargo-door information and compare it with the wheelchair’s travel-ready dimensions instead of assuming that every wheelchair is compatible with every aircraft.

Recommended Database Fields

- Aircraft manufacturer

- Aircraft model/type

- Cargo door width (cm)

- Cargo door height (cm)

- Aircraft/mobility-aid weight limitation

- Wheelchair travel-ready length (cm)

- Wheelchair travel-ready width (cm)

- Wheelchair travel-ready height (cm)

- Fits through cargo door — Yes/No

- Aircraft compatible — Yes/No

- Special loading or handling requirements

- Compatibility notes

- Aircraft-dimension information source/reference

Why This Information Is Needed

Comparing the wheelchair’s travel-ready dimensions with the cargo-door dimensions of a specific aircraft can help identify compatibility problems before the passenger arrives at the airport. This allows airlines and passengers to plan suitable aircraft or alternative travel arrangements where necessary.

Database Recommendation

Based on the research, the YAF database should mainly store three groups of information:

Mobility Aid

- Manufacturer and model

- Type

- Travel-ready length, width and height

- Total weight

- Removable/adjustable parts

Battery

- Battery type

- Watt-hour (Wh) rating where applicable

- Battery removable — Yes/No

- UN 38.3 compliance for lithium batteries

- Power isolation instructions

Aircraft

- Aircraft model/type

- Cargo door width and height

- Weight limitation

- Compatibility result — Compatible / Review Required / Not Compatible

These fields would allow YAF to compare a mobility aid's travel-ready specifications with aircraft limitations and identify potential transport issues before travel.

### Source

  Mobility Aids (Wheelchairs) — Battery Checklist (IATA 2026). Aligned to ICAO Technical Instructions 2025–2026 and IATA Dangerous Goods Regulations, 67th Edition (2026). https://www.dgonline.training/iata-2026-guidance-battery-powered-wheelchairs-mobility-aids-updates/

  International Air Transport Association (IATA). (2023). Guidance on the Transport of Mobility Aids – First Edition, February 2023. https://www.iata.org/contentassets/7b3762815ac44a10b83ccf5560c1b308/iata-guidance-on-the-transport-of-mobility-aids-final-feb2023.pdf